CRM62 / CRM84 RS-232 control guide

Revision 1.2

3/1/06

Overview

The RS-232 port on the CRM series switches is a flexible interface which can easily control all aspects of the device operation.  Many system integrators prefer RS-232 control over IR because it allows the controlling device to monitor the status of the CRM switch.

Hardware specifics

The CRM switch uses a DB 9 female connector (shown below).

Only 3 pins on the connector are needed:

Pin 2:  TX  (from CRM to controller)

Pin 3:  RX  (from controller to CRM)

Pin 5:  Ground

The cable used should be a standard straight through RS-232 cable with a DB9 male plug (also known as a serial modem cable).

Protocol settings

Baud rate : 9600

Data bits: 8

Stop bits: 1

Parity: None

Flow control: None

Commands

CRM62/84 will echo back all characters sent to it.  This is the easiest way to verify if your cable connection is correct.  The CRM62/84 does not add <lf> after any <cr> received, so if you are using windows Hyper-terminal you should change the default settings to allow line feed on carriage return.

All commands are terminated by <cr> (carriage return, ascii code 0xD).  The following is the valid command set for CRM62/84.  Any commands that are not recognized will be interpreted as a status request.

Command set:

V##(#)<cr> :	Video output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

3rd # only present in basic scheme  (since audio is tied to video)

D selects digital coax, O selects digital toslink

A###<cr> :  Audio output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

3rd # D selects digital coax, O selects digital toslink

H0##<cr> :  Digital video control

1st # selects the input to switch the output to

2nd # D turns boost off, O turns boost on

C##<cr> :  Config control

1st # selects which control you will change

2nd # F control off, T turns control on

Control 1 : Clear ZeroKey IR codes  (T runs clear, auto sets back to F)

Control 2 : Autoswitch for digital video off

Control 3:  Advanced scheme enable

Control 4:  Power off

Control 5:  LED off

STAT<cr> :  Request device status

Response strings

The CSM62/84 has two possible response strings:

- ACKNOWLEDGE   (OK<cr>)   :  This is sent to indicate the previous command was executed

- STATUS    This is sent whenever explicitly requested OR when an invalid command is received

The format of the status response is:

Video Storm LLC CRM switch<cr><cr>

V1#<cr>

V2#<cr>

(V3#)<cr>		Present if CRM84

(V4#)<cr>		Present if CRM84

A1##<cr>

A2##<cr>

(A3##)<cr>		Present if CRM84

(A4##)<cr>		Present if CRM84

(H##)<cr>		Present if digital video board installed

C2#<cr>

C3#<cr>

C4#<cr>

C5#<cr>

The format of the status strings is the same as the command strings before.  V is video, A is audio, H is digital video, and C is config.

The first # denotes the input selected.  For audio second # is D for digital coax or O or digital optical.  For digital video (H), D means boost off & O means boost on.

Example 1:  V34  =>  Video output 3 has input 4 selected

Example 2:  A17O =>  Audio output 1 has input 7 selected with TOSLINK

The config settings are interpreted as follows:

C2T || C2F	Autoswitch enabled when F

C3T || C3F	Advanced scheme enabled when T

C4T || C4F	Power on when F

C5T || C5F	LED on when F