CRM88 RS-232 control guide

Revision 1.0

3/1/07

Overview

The RS-232 port on the CRM series switches is a flexible interface which can easily control all aspects of the device operation.  Many system integrators prefer RS-232 control over IR because it allows the controlling device to monitor the status of the CRM switch.

Hardware specifics

The CRM switch uses a DB 9 female connector (shown below).

Only 3 pins on the connector are needed:

Pin 2:  TX  (from CRM to controller)

Pin 3:  RX  (from controller to CRM)

Pin 5:  Ground

The cable used should be a standard straight through RS-232 cable with a DB9 male plug (also known as a serial modem cable).

Protocol settings

Baud rate : 9600

Data bits: 8

Stop bits: 1

Parity: None

Flow control: None

Commands

CRM88 will echo back all characters sent to it.  This is the easiest way to verify if your cable connection is correct.  The CRM88 does not add <lf> after any <cr> received, so if you are using windows Hyper-terminal you should change the default settings to allow line feed on carriage return.

All commands are terminated by <cr> (carriage return, ascii code 0xD).  The following is the valid command set for CRM88.  Any commands that are not recognized will be interpreted as a status request.

Note the command set is different in 8x8 basic, 8x8 advanced, and 16x16 mode.  Config controls present in all modes.

Command set (config controls):

CF##<cr> :  Config control

1st # selects which control you will change

2nd # F control off, T turns control on

Control 1:  Clear ZeroKey IR codes & config bits  (T runs clear, auto sets   back to F)

Control 2:  LED off

Control 3:  Power off

Control 4:  Advanced scheme enable (only has effect in 8x8 mode)

Control 5:  Video/Audio mode (F is video, T is audio : only has effect if in  16x16 mode.   Note that switching audio/video mode also requires a jumper change for the input termination)

Control 6:  16x16 mode enable

STAT<cr> :  Request device status

Command set (8x8 basic mode):

V###<cr> :	All output control   (everything tied to video)

1st # selects the desired output to control

2nd # selects the input to switch the output to

3rd #  D selects digital coax, O selects digital toslink

Command set (8x8 advance mode):

V##<cr> :	Component video output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

A##<cr> :  Audio output control     (advanced mode only)

1st # selects the desired output to control

2nd # selects the input to switch the output to

P##<cr> :  Composite video output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

C###<cr> :  Digital audio coax output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

3rd #  D selects digital coax, O selects digital toslink

O###<cr> :  Digital audio toslink output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

3rd #  D selects digital coax, O selects digital toslink

Command set (16x16 mode):

####<cr> :	16x16 output control

1st & 2nd # selects the desired output to control (01, 02, …, 16)

3rd & 4th # selects the input to switch the output to (01, 02, …, 16)

C###<cr> :  Digital audio coax output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

3rd #  D selects digital coax, O selects digital toslink

O###<cr> :  Digital audio toslink output control

1st # selects the desired output to control

2nd # selects the input to switch the output to

3rd #  D selects digital coax, O selects digital toslink

Response strings

The CRM88 has two possible response strings:

- ACKNOWLEDGE   (OK<cr>)   :  This is sent to indicate the previous command was executed

- STATUS    This is sent whenever explicitly requested OR when an invalid command is received

The format of the status response is different in 8x8 and 16x16 modes  8x8 format is:

Video Storm LLC CRM switch<cr><cr>

V1#<cr>

V2#<cr>

(V3-V8)		Same as above

A1#<cr>

(A2-A8)

P1#<cr>

(P2-P8)

C1##<cr>

(C2-C8)

O1##<cr>

(O2-O6)

CF2#<cr>

CF3#<cr>

CF4#<cr>

CF5#<cr>

CF6#<cr>

The format of the status strings is the same as the command strings before.  V is video, A is analog audio, P is composite video, C is digital audio coax, O is digital audio toslink, and CF is config.

The first # denotes the input selected (1-8).  For digital audio, the second # is D for digital coax or O or digital optical.

Example 1:  V34  =>  Component video output 3 has input 4 selected

Example 2:  C16O =>  Coax audio output 1 has toslink audio input 6 selected

16x16 format is:

Video Storm LLC CRM switch<cr><cr>

01##<cr>

02##<cr>

(03-16)		Same as above

C1##<cr>

(C2-C8)

O1##<cr>

(O2-O6)

CF2#<cr>

CF3#<cr>

CF4#<cr>

CF5#<cr>

CF6#<cr>

The format of the status strings is the same as the command strings before.  01-16 is the output select, C is digital audio coax, O is digital audio toslink, and CF is config.

For the 1-16 outputs, ## denotes the input selected (01-16).  For digital audio, the first # is input selected (1-8) and the second # is D for digital coax or O or digital optical.

Example 1:  1304  =>  Output 13 has input 4 selected

Example 2:  C16O =>  Coax audio output 1 has toslink audio input 6 selected

The config settings are interpreted as follows:

C2T || C2F	LED on when F

C3T || C3F	Power on when F

C4T || C4F	User mode when F, Advanced mode when T

C5T || C5F	Video mode when F, Audio mode when T  (only has effect when in 16x16)

C6T || C6F	8x8 mode when F, 16x16 when T