Model CMX86

Ultra Wide Bandwidth HDTV

Matrix Switch

(firmware 1.9)

Overview:

This product is a full featured video & audio matrix switch.  It is most commonly used to independently distribute video & audio to several different viewing areas from a set of common sources.  CMX86 provides 6 output zones from 8 common sources.

CMX86 has 2 operational modes:

- 8x6 basic control mode:  (all video and audio types switch together)

- 8x6 advanced control mode:  (all video and audio types controlled separately)

The matrix switch has 3 control modes to choose from:

- Manual control from the front buttons

- IR control by remote control

- Serial control via the RS-232 port (db9).

Unit includes

- CMX86 hdtv matrix switch

- Owners manual

- 12V desktop power supply

- 3 year parts and labor warranty

- Optional rack mounting ears & screws

Warranty

Video Storm LLC products are covered by our manufacture’s warranty.   Our warranty covers parts and labor for 3 years following purchase.  In the unlikely event you receive an item that was not damaged in shipping but is defective please contact support@video-storm.com and we will repair or replace the item promptly.

Installation

Please use the following steps to install and set up your CMX86 in the default 8x6 basic mode.

When looking at the rear of the CMX86, all inputs are on the left half and all outputs are on the right half.

- Component video:  Connect the component video sources to the input red/blue/green RCA jacks marked 1-8.  Connect the output cables to the output red/blue/green RCA jacks marked 1-6.  Be sure to match the cable colors to RCA jack color.

- Analog audio:  Connect the analog stereo audio sources to the input red/white RCA jacks marked 1-8.  Connect the output cables to the output red/white RCA jacks marked 1-6.  Be sure to match the cable colors to RCA jack color.

- Composite video:  Connect the composite video sources to the input yellow RCA jacks marked 1-8.  Connect the output cables to the output yellow RCA jacks marked 1-6.  Note that there are 2 different sets of yellow RCA jacks.  The jacks next to the red/white jacks are composite video.

- Digital coax audio:  Connect the digital audio sources to the yellow RCA jacks marked 1-8.  Connect the output cables to the output yellow RCA jacks marked 1-6.  Note that there are 2 different sets of yellow RCA jacks.  The jacks near the TOSLINK connectors are digital audio coax.

- Digital TOSLINK audio:  Connect the digital audio sources to the input TOSLINK jacks marked 1-6.  Connect the output cables to the output TOSLINKS jacks marked 1-6.

- RS-232:  If needed, connect your computer or controller to the DB9 jack labeled “Control” using a DB9 male to female normal modem cable.

- RS-232 expansion:  If using a HMX HDMI matrix, connect a DB9 male to female normal modem cable from the RS-232 jack labeled “XP” to the RS-232 jack on the HMX matrix.

- Connect the supplied wall transformer to a properly grounded power outlet.  Plug into DC input of the CMX86.

- Press one of the front panel buttons; you should see the LED display turn on.

Controlling the CMX86

The CMX86 can be controlled by 3 methods:  front panel switch, infrared remote, or RS-232 serial.  You may use any or all methods in any combination.  All methods can access all features and modes.

The left front panel button controls the output selection (left two LED digits).  Each press will cycle through the available outputs.  The right button controls the input selection (right two LED digits).  Each press will cycle through the available inputs.  Pressing and holding the left button for 3 or more seconds will jump the output selection to the next connector type when released.  Pressing and holding the right button for 3 or more seconds is only used in Zerokey programming mode, config mode, for volume mute toggle, or for quick select mode.

NOTE: Output needs to be selected before setting the input.

Quick select can be used to easily send a single input to all outputs.  To use quick select, simply select output 1 using the left button, toggle to the input desired using the right button, and then press and hold the right button for 3 or more seconds.

Flash modes allow the user to save and later recall up to 8 different switch setups.  Flash modes are saved to flash memory, so are unaffected by power cycling.  To use the flash modes, select “Fx” (x is 1-8) using the left button (or IR).  The right LED will display “E” if no saved mode exists in this slot, otherwise “F”.  Use a short press with the right button to recall these settings.  Use a long press with the right button to save the current switch setup to this flash slot.  When using IR, the sequence is (Output type select “F”, input select 0-15).  Use input select codes 0-7 to recall flash modes 1-8, and input select codes 8-15 to save to flash modes 1-8.

The IR remote and RS-232 interface can select outputs and inputs directly.  If using the RS-232 interface, please refer to the programming guide on our web site.   The IR interface is explained later in this document.

The table below explains what the LED displays indicate.  Note that generally the left two digits display the selected output and the right two digits display the selected input.  When selecting an output greater than 9, the letter will flash for 1 second and then the “1” will be displayed.  When “0” is displayed as the selected input that output is powered off.

Basic mode

| Left LED Display (selected output) | Right LED Display (selected input) | Description |
| --- | --- | --- |
| U 1-6 | C 0-8 or O 1-8 | Control for all outputs types EXCEPT HDMI For the right display, when C is shown the coax digital input is transcoded to both coax and TOSLINK outputs. When O is shown, the TOSLINK digital input is transcoded to both outputs |
| H 1-16 | 0-16 | Control for HDMI outputs (if HMX switch is connected) |
| G 1-6 | 0-16 | Volume control for analog audio outputs. G1 corresponds to analog audio output 1. The right display is the volume, “00” is muted while “16” is max volume. A “.” After the number indicates MUTE is ON |
| E 1-8 | 0-15 | Volume control for analog audio inputs. E1 corresponds to analog audio input 1. The right display is the volume, “00” is min while “15” is max volume. |
| B 1-8 | 0-15 | Volume balance for analog audio outputsThe right display is the balance, “08” nominal. |
| F 1-8 | E/F | Flash mode slot 1-8. If right LED shows E, then slot is empty. If right LED shows F, then slot is active. |
| CF | C-U , O/F | Config menu, please see setting below |
| SZ | 0-40 | Zerokey programming mode, please see Zerokey section |

Advanced mode

| Left LED Display (selected output) | Right LED Display (selected input) | Description |
| --- | --- | --- |
| U 1-6 | 0-8 | Component video control |
| A 1-6 | 0-8 | Analog audio control |
| P 1-6 | 0-8 | Composite video control |
| C 1-6 | C 0-8 or O 1-6 | Digital audio coax control. When C is shown on the right it means you are selecting the coaxial audio input. When O is display you are selecting TOSLINK audio input. |
| O 1-6 | C 0-8 or O 1-6 | Digital audio TOSLINK control. When C is shown on the right it means you are selecting the coaxial audio input. When O is display you are selecting TOSLINK audio input. |
| H 1-16 | 0-16 | Control for HDMI outputs (if HMX switch is connected) |
| G 1-6 | 0-16 | Volume control for analog audio outputs. G1 corresponds to analog audio output 1. The right display is the volume, “00” is muted while “16” is max volume. A “.” After the number indicates MUTE is ON |
| E 1-8 | 0-15 | Volume control for analog audio inputs. E1 corresponds to analog audio input 1. The right display is the volume, “00” is min while “15” is max volume. |
| B 1-8 | 0-15 | Volume balance for analog audio outputs. The right display is the balance, “08” nominal. |
| F 1-8 | E/F | Flash mode slot 1-8. If right LED shows E, then slot is empty. If right LED shows F, then slot is active. |
| CF | C-U, O/F | Config menu, please see setting below |
| SZ | 0-41 | Zerokey programming mode, please see Zerokey section |

Right LED display for configuration menu:

- C  	Clear memory.  Press the right front panel key and hold for 6 seconds to reset all Zerokey IR codes to factory defaults and put the switch into basic mode.

- L	LED OFF.  Press and hold the right switch to black out the LED display.  Any button will restore.

- P	Power OFF.  Press and hold the right switch to turn off CMX86.  Any button will restore power.

- U O/F	Controls basic or advanced mode selection.  Press & hold the right switch to turn on/off .  F means basic mode, O means advanced mode

- 1 O/F	HDMI detection (O is on, F is off).  Press & hold the right switch to turn on/off

- 2 O/F	Auto refresh (O is on, F is off).  Press & hold the right switch to turn on/off

- 3 O/F	Sets power on default selections.  When this control is switched from F to O it saves the current state of all input / output selections to flash memory.  Whenever the CMX86 powers up it will revert to these saved settings.  If this control is set to F the power default will be all outputs disabled.

- 4 O/F	Disable audio gain functions (when set to O)

IR remote interface

The CMX86 IR interface consists of 43 discrete IR codes.  These codes are stored in internal flash memory.  When the CMX86 detects a transmitted IR code which matches one or more of the codes in memory, it performs the action dictated by that code.  The table below shows what action each code performs.  The factory defaults for all 43 codes can be obtained in CCF format from our web site.  If you have a fully programmable remote, you can load these codes into the remote.  If your remote does not support loading hex IR codes, then you will need to program new codes into the CMX86.  See the section on programming the ZeroKey interface for instructions.

| IR code | Action performed |
| --- | --- |
| 0 | Selects the output type “U” (component video in advanced mode or all types except HDMI in basic mode) |
| 1 | Selects the output type “A” (analog audio) |
| 2 | Selects the output type “P” (composite video) |
| 3 | Selects the output type “C” (coax digital audio) |
| 4 | Selects the output type “O” (TOSLINK audio) |
| 5 | Selects the output type “H” (HDMI) |
| 6 | Selects the output type “G” (audio gain) |
| 7 | Selects the output type “CF” (config menu) |
| 8-23 | Specific output select 1-16. The output selected is the (IR code – 7) |
| 24-41 | Input select 0-17. The input selected is the (IR code – 24) |
| 42 | Selects the output type “F” (flash modes) |
| 43 | Selects the output type “B” (audio balance) |

The sequence for switching an output requires sending 3 IR codes.  First send the code to select the output type (code 0-7), then the code to select the specific output (code 8-23), then the code to select the input (code 24 – 40).   For example (in advanced mode), to switch analog audio output 3 to input 5 send codes 1, then 10, then 29.

Special considerations:

- Basic mode:  All output types except HDMI are selected with output type code 0.  Input selects 0-8 (codes 24-32) select coaxial digital audio.  Input selects 9-16 (codes 33-40) select TOSLINK digital audio.

- Audio gain:  The input select codes control volume instead.  “0” is mute toggle and “15” is max volume. Code “16” is Volume up (increases volume each press).  Code “17” is Volume down.

- Set ZeroKey mode cannot be accessed by IR or RS-232.  This must be done by front panel buttons if necessary (one time only).

- Config menu:  The specific output select code is not used (ignored).  The input select codes work as follows.

| Input select (IR code) | Action |
| --- | --- |
| 1 (25) | Turns LED OFF |
| 9 (33) | Turns LED ON |
| 2 (26) | Powers OFF CMX |
| 10 (27) | Powers ON CMX |
| other | All other codes should not be used when in config menu |

Programming the ZeroKey remote interface

The remote control interface can use ZeroKey infrared code cloning to map any remote control key to each of the 42 IR control codes.  Depending on how you are using CMX86, you may not need to program every IR code.  For example, if you are using CMX86 in basic mode without HDMI, you only need to program IR codes 0, 8-14, and 9 of 24-40 depending on which digital audio types you are using (16 total codes).

Before starting, you will need a remote with as many unique “keys” as codes you wish to program.  Keep in mind; most universal remotes have 5-6 “pages” of codes.  For example, when you select “VCR” control the numeric keys on the remote have different IR codes than when you select “DVD” on the remote.

Here are the steps to program the CMX86 Zerokey control codes for your remote.

- Toggle the left button until “SZ” is displayed on the left LED

- The right LED will display a number 0-41.  This corresponds to the IR code selected.

- Toggle the right button to the IR code you wish to program

- Press and hold the right button for 6 seconds.  When released the right LED should start to flash.

- Press the remote button you wish to map to this IR code

- The right LED should stop flashing, indicating the code was stored.

- The CMX may switch at this point if programming IR codes 0-7.  This is normal, just toggle back to the “SZ” mode.

- Repeat for all other IR codes you wish to use.

- When done, test every IR code.  If one or more do not work repeat the programming for those codes (sometimes interference will cause the code learn to fail).  If you cannot get a specific code to work after multiple attempts, try using a different remote key.

For additional examples of programming the ZeroKey IR interface, please refer to the quick start guide.

ZeroKey technology is compatible with virtually all remote controls, and does not interfere with the normal operation of the remote.  You may reset all the ZeroKey codes to the factory defaults by selecting “CFC” and holding the input button for 6 seconds.  ZeroKey programming is stored in flash memory, and you will not need to reprogram even if you lose power.  If you buy a new remote in the future, don't worry, the CMX86 can be reprogrammed as much as you like.

Output AC / DC coupling

The CMX86 has configurable output coupling for the best in display compatibility.  All outputs are normally set to AC coupling at the factory.  If you encounter loss of sync during scene transitions on one of your displays, you should change that output to DC coupling to resolve the issue.  Please follow the instructions on the CMX86 product page at www.video-storm.com.

Specifications

- 32 High bandwidth analog inputs  (used for component video and composite video)
    75Ohm terminated at all frequencies
    Gold plated matched low impedance RCA jacks
    1Vpp nominal input swing (Ypbpr) or 1.3Vpp (Cvbs)   4Vpp maximum  (DC invariant)
    DC rejection via capacitive coupling
    Crosstalk < -80 dB

- 24 High bandwidth analog outputs  (used for component video and composite video)
   Source impedance 75 Ohms
   Gold plated matched low impedance RCA jacks
   All output sets individually controllable/switchable
   Selectable AC or DC coupling
   High performance amplifier capable of driving over 300ft of coax cable per output
  2V/V (video) gain to properly drive output load termination (zero effective gain at display)
   -3dB bandwidth > 100 Mhz

- 16 Analog audio inputs
    10KOhm terminated at all frequencies
    Gold plated matched low impedance RCA jacks
    2.6Vpp nominal input swing    4Vpp maximum  (DC invariant)
    DC rejection via capacitive coupling 
    256 step programmable input volume equalization
    Crosstalk < -110 dB

- 12 Analog audio outputs
   Source impedance 75 Ohms
   Gold plated matched low impedance RCA jacks
   All output sets individually controllable/switchable
   High performance Burr-Brown amplifier capable of driving over 300ft of coax cable per output
   256 step programmable volume control
   THD+N < 0.0004%

- 6 Optical Digital audio inputs + 6 Optical Digital audio outputs
   Optical digital audio inputs have built in shutters, no plugs needed
   Selectable transcoding from coax digital audio
   All outputs individually controllable/switchable

- 8 Coax Digital audio inputs + 6 Coax Digital audio outputs
   Gold plated matched low impedance RCA jacks
   AC coupled to protect all connected equipment
   Capable of driving over 300ft of coax cables
   Selectable transcoding from optical digital audio 
   All outputs individually controllable/switchable

- IR receiver (front)

- RS-232 control input (DB9 female) & RS-232 expansion output (DB9 male)

- Internal design 
    Flash programmable micro controller enables ZeroKey remote technology
    Ultra wide bandwidth integrated buffered t-switch matrix prevents crosstalk or signal distortion
    Independent professional quality video amplifiers for best performance
    High reliability capacitors and self resetting fuse protection for long life
    Designed and manufactured in the USA

- Housing 
    Black steel rack mountable housing   17" x 13" x 3.5"   (2 RU)