Quick setup guide:		CRM62

Example system (only using 4 inputs, CRM62 supports up to 6):

Cable box with component video, analog audio, and digital audio coax out

Xbox with component video, analog audio, and digital audio optical out

DVD with component video, analog audio, and digital audio optical out

PVR with component video, analog audio, and digital audio optical out

Projector (living room) with component video in

Receiver (living room) with analog audio and digital audio optical in

Plasma (bed room) with component video in and analog audio in

Receiver (bed room) with analog audio and digital audio coax in

Step 1:  Plan your system

What we are trying to accomplish is a full featured dual zone viewing environment.  We will be able to watch any of the 4 sources on each display.  Since CRM62 is a matrix switch, we can display the same source to both viewing areas or different sources.

For audio, we will be sending analog audio and digital optical to the living room.  Analog audio and digital coax will be sent to the bed room.  We are sending both analog audio and digital audio to the bed room so we can use the Plasma speakers only if we do not want to turn on the receiver.  CRM62 will transcode between digital audio coax and optical, so we can use coax in the bedroom since it is a less expensive long cable.  We will use optical in the living room, but could switch to coax if our needs change.

For simplicity, we will use the basic scheme => audio and video for each output switch together.  If in the future we decide that we would like to control the audio and video separately, we can always change to advanced scheme.

For control, we will use both the front panel buttons and the IR remote.  If we had a fully programmable remote like the Pronto, we could use the default IR codes and just load the config file into the Pronto.  Here we do not have a Pronto, so we will use the Zerokey IR learning function of the CRM62 with our basic remote.

Once all the cables are in place, we are ready to install the CRM62

Step 2:  Connecting CRM62

Make sure all the equipment is unplugged.  See the figure on the last page for illustration.

Cable box:

- Connect the component video cable from the red/green/blue rca jacks on the cable box to the red/green/blue jack on CRM62 labeled “1”

- Connect the analog audio cable from the red/white rca jacks on the cable box to the red/white jack on CRM62 labeled “1”

- Connect the digital audio cable from the correct rca jack on the cable box to the yellow jack on CRM62 labeled “1”

Xbox:

- Connect the component video cable from the red/green/blue rca jacks on the Xbox hd output adapter to the red/green/blue jack on CRM62 labeled “2”

- Connect the analog audio cable from the red/white rca jacks on the Xbox hd output adapter to the red/white jack on CRM62 labeled “2”

- Connect the toslink cable from the toslink jack on the Xbox hd output adapter to the toslink jack on CRM62 labeled “2”

DVD:

- Connect like Xbox using the CRM62 jacks labeled “3”

PVR:

- Connect like Xbox using the CRM62 jacks labeled “4”

Projector:

- Connect the component video cable from the red/green/blue rca jacks on the projector to the red/green/blue jack on CRM62 labeled “OUT 1”

Receiver (living room):

- Connect the toslink cable from the toslink jack on the receiver to the toslink jack on CRM62 labeled “O1”

Plasma:

- Connect the component video cable from the red/green/blue rca jacks on the plasma to the red/green/blue jack on CRM62 labeled “OUT 2”

- Connect the analog audio cable from the red/white rca jacks on the plasma to the red/white jack on CRM62 labeled “OUT 2”

Receiver (bed room):

- Connect the digital audio cable from the correct rca jack on the receiver to the yellow jack on CRM62 labeled “OUT 2”

Step 3:  Configuring the scheme

Plug in the CRM62 and press on of the front buttons to power it on

(Note basic scheme is the factory default, so this step is only shown for example)

Toggle the left button until “C” (Config) is displayed on the left LED

Toggle the right button until “U” or “U.” (basic or advanced scheme) is displayed on the right LED

If “U.” (advanced scheme) is displayed, press and hold the right button for 5 seconds

The LED should now show “U” (basic scheme), verifying you are in basic scheme

Step 4:  Controlling CRM62 via the front panel buttons

The left button will select which output you are controlling or config mode.  The left button display is the left LED.

“1” : Output 1 is selected, the right LED will now show the current input for output 1

“2” : Output 2 is selected, the right LED will now show the current input for output 2

“C” : Config mode, the right LED will now show various config controls

The right button will toggle the right LED.  Use this button to select the input which will be connected to the current output.  The right LED will toggle through “1-6”, and the “1.-6.”.  When the “.” Is displayed, the digital audio optical jack input is being transcoded to both digital audio out coax & optical.  When the “.” Is not displayed, the digital audio coax jack input is being transcoded to both digital audio out coax & optical.

Selecting DVD to living room projector & receiver:

Toggle left button until the left led shows “1”

Toggle right button until the right led shows “3.”

Selecting DVD to bed room plasma & receiver:

Toggle left button until the left led shows “2”

Toggle right button until the right led shows “3.”

Selecting Cable box to bed room plasma & receiver:

Toggle left button until the left led shows “2”

Toggle right button until the right led shows “1”

Selecting Cable box to living room projector & receiver:

Toggle left button until the left led shows “1”

Toggle right button until the right led shows “1”

Step 5:  Using ZeroKey IR learning for remote control

The IR interface of the CRM62 is very similar to the front panel control.  We are just substituting a unique IR code (or remote button) for each of the left and right LED possible values.  For CRM62, this gives us 14 discreet codes in basic scheme:

Output select codes:  1, 2

Input select codes: 1, 2, 3, 4, 5, 6, 1., 2., 3., 4., 5., 6.

To control the CRM62, we can to map each of these discreet codes to a separate key on our remote.  However, since in our setup we are only using inputs 1, 2., 3., 4. we only need to use six remote keys.

We are going to start by finding our receiver universal remote.  Like most universal remotes, it can control several devices.  In our system, we don’t have a VCR so we are going to use those codes to control CRM62.

Remote (VCR selected) key 1 => output select 1 (projector)

Remote (VCR selected) key 2 => output select 2 (plasma)

Remote (VCR selected) key 3 => input select 1 (cable)

Remote (VCR selected) key 4 => input select 2. (xbox)

Remote (VCR selected) key 5 => input select 3. (dvd)

Remote (VCR selected) key 6 => input select 4. (pvr)

For each of these remote keys, we need to program the CRM62 to learn the transmitted code.  Keep in mind it is the CRM62 that is doing the recording, so don’t try to put the remote in “learn” mode.  Here are the steps for each key:

For the output selects (1 and 2)

Toggle the left front panel button until the correct output is displayed

Press and hold the left front panel button for 5 seconds

Release the left front panel button

The left LED should now be flashing, it is ready to record

Press the correct key on the remote (see above table)

The left LED should stop flashing, this key is stored

For the input selects (1-4.)

Toggle the right front panel button until the correct input is displayed

Press and hold the right front panel button for 5 seconds

Release the right front panel button

The right LED should now be flashing, it is ready to record

Press the correct key on the remote (see above table)

The right LED should stop flashing, this key is stored

After storing all 6 remote keys, you can now control CRM62 with that remote.

Selecting DVD to projector:

Make sure universal remote is on “VCR” setting

Press remote button 1  (select projector)

Press remote button 5  (select DVD)

Selecting Cable box to plasma:

Make sure universal remote is on “VCR” setting

Press remote button 2  (select plasma)

Press remote button 3  (select Cable)

If you ever need to change remotes, you can use the config clear command (“CC” on LED and hold right button for 3 seconds) to reset all the codes at once.

This completes installation and setup of the CRM62.  If you have further questions please check our FAQ forum at video-storm.com for the most recent information

Rear panel diagram: