Quick setup guide:		CRM88

Example system (only using 4 inputs, CRM88 supports up to 8 in 8x8 mode):

Sources:

1. Cable box with component & composite video, analog audio, and digital audio coax out

2. Xbox with component video, analog audio, and digital audio optical out

3. DVD with component & composite video, analog audio, and digital audio optical out

4. PVR with component & composite video, analog audio, and digital audio optical out

Displays:

1. Projector (living room) with component video in

Receiver (living room) with analog audio and digital audio optical in

2. Plasma (bed room) with component video in and analog audio in

Receiver (bed room) with analog audio and digital audio coax in

3. RPTV (game room) with component video in and analog audio in

Receiver (game room) with analog audio and digital audio coax in

4. LCD (kitchen) with component video in and analog audio in

5. CRT (garage) with composite video in and analog audio in

Step 1:  Plan your system

What we are trying to accomplish is a full featured 5 zone viewing environment.  We will be able to watch any of the 4 sources on each display.  Since CRM88 is a matrix switch, we can display the same source to both viewing areas or different sources.

For audio, we will be sending analog audio and digital optical to the living room.  Analog audio and digital optical will be sent to the bed room and game room.  We are sending both analog audio and digital audio so we can use the tv speakers only if we do not want to turn on the receiver.  For the kitchen, we will send analog audio only since we don’t have a receiver.  CRM88 will transcode between digital audio coax and optical, so we can use coax in the bedroom and game room since it is a less expensive long cable.  We will use optical in the living room, but could switch to coax if our needs change.  For the garage we will send composite video and analog audio.

For simplicity, we will use the basic 8x8 mode => audio and video for each output switch together.  If in the future we decide that we would like to control the audio and video separately, we can always change to advanced mode.  We don’t have enough displays to need 16x16 mode right now, but it is there if we eventually need it.

For control, we will use both the front panel buttons and the IR remote.  If we had a fully programmable remote like the Pronto, we could use the default IR codes and just load the config file into the Pronto.  Here we do not have a Pronto, so we will use the Zerokey IR learning function of the CRM88 with our basic remote.

Once all the cables are in place, we are ready to install the CRM88

Step 2:  Connecting CRM88

Make sure all the equipment is unplugged.  See the figure on the last page for illustration.

Remember, all CRM88 inputs are on the LEFT side when looking at the back!

Cable box:

- Connect the component video cable from the red/green/blue rca jacks on the cable box to the red/green/blue input jack on CRM88 labeled “1”

- Connect the composite video cable from the yellow jack on the cable box to the yellow composite video input jack on CRM88 labeled “1”

- Connect the analog audio cable from the red/white rca jacks on the cable box to the red/white input jack on CRM88 labeled “1”

- Connect the digital audio cable from the correct rca jack on the cable box to the yellow digital audio input jack on CRM88 labeled “1”

Xbox:

- Connect the component video cable from the red/green/blue rca jacks on the Xbox hd output adapter to the red/green/blue input jack on CRM88 labeled “2”

- Connect the analog audio cable from the red/white rca jacks on the Xbox hd output adapter to the red/white input jacks on CRM88 labeled “2”

- Connect the toslink cable from the toslink jack on the Xbox hd output adapter to the toslink input jack on CRM88 labeled “2”

DVD:

- Connect like Xbox using the CRM88 input jacks labeled “3”.

- Connect the composite video cable from the yellow jack on the DVD to the yellow composite video input jack on CRM88 labeled “3”

PVR:

- Connect like DVD using the CRM88 input jacks labeled “4”.

Remember, all CRM88 outputs are on the RIGHT side when looking at the back!

Projector:

- Connect the component video cable from the red/green/blue rca jacks on the projector to the red/green/blue output jack on CRM88 labeled “1”

Receiver (living room):

- Connect the toslink cable from the toslink jack on the receiver to the toslink output jack on CRM88 labeled “1”

Plasma:

- Connect the component video cable from the red/green/blue rca jacks on the plasma to the red/green/blue output jacks on CRM88 labeled “2”

- Connect the analog audio cable from the red/white rca jacks on the plasma to the red/white output jacks on CRM88 labeled “2”

Receiver (bed room):

- Connect the digital audio cable from the correct rca jack on the receiver to the yellow digital audio output jack on CRM88 labeled “2”

RPTV:

- Connect the component video cable from the red/green/blue rca jacks on the RPTV to the red/green/blue output jacks on CRM88 labeled “3”.

- Connect the analog audio cable from the red/white rca jacks on the RPTV to the red/white output jacks on CRM88 labeled “3”.

Receiver (game room):

- Connect the digital audio cable from the correct rca jack on the receiver to the yellow digital audio output jack on CRM88 labeled “3”.

LCD:

- Connect the component video cable from the red/green/blue rca jacks on the LCD to the red/green/blue output jacks on CRM88 labeled “4”.

- Connect the analog audio cable from the red/white rca jacks on the LCD to the red/white output jacks on CRM88 labeled “4”.

CRT:

- Connect the composite video cable from the yellow rca jack on the CRT to the yellow composite video output jack on CRM88 labeled “5”.

- Connect the analog audio cable from the red/white rca jacks on the CRT to the red/white output jacks on CRM88 labeled “5”.

Step 3:  Configuring the mode

Plug in the CRM88 and press on of the front buttons to power it on

(Note basic mode is the factory default, so this step is only shown for example)

Toggle the left button until “CF” (Config) is displayed on the left LED

Toggle the right button until “UF” or “UO” (basic or advanced mode) is displayed on the right LED

If “UO” (advanced scheme) is displayed, press and hold the right button for 6 seconds

The LED should now show “UF”, verifying you are in basic mode

Now toggle the right button until “HF” or “HO” is displayed on the right LED (left LED should still be “CF”)

If “HO” (16x16 mode) is displayed, press and hold the right button for 6 seconds

The LED should now show “HF”, verifying you are in 8x8 mode

Note:  You could instead select “C” on the right LED and then press and hold the right button for 6 seconds.  This would set CRM88 back to factory defaults, which is 8x8 basic mode.

Step 4:  Controlling CRM88 via the front panel buttons

The left button will select which output you are controlling or config mode.  The left button display is the left LED.

“U1” : Output 1 is selected, the right LED will now show the current input for output 1

“U2” : Output 2 is selected, the right LED will now show the current input for output 2

“U3” : Output 3 is selected, the right LED will now show the current input for output 3

“U4” : Output 4 is selected, the right LED will now show the current input for output 4

“U5” : Output 5 is selected, the right LED will now show the current input for output 5

“U6-8” : Output 5-8 selections.  We do not need these.

“CF” : Config mode, the right LED will now show various config controls

The right button will toggle the right LED.  Use this button to select the input which will be connected to the current output.  The right LED will toggle through “1C-8C”, and then “1O-8O”.  When “O” is displayed, the digital audio optical jack input is being transcoded to both digital audio out coax & optical.  When “C” is displayed, the digital audio coax jack input is being transcoded to both digital audio out coax & optical.

Selecting DVD to living room projector & receiver:

Toggle left button until the left led shows “U1”

Toggle right button until the right led shows “3O”

Selecting DVD to bed room plasma & receiver:

Toggle left button until the left led shows “U2”

Toggle right button until the right led shows “3O”

Selecting Cable box to bed room plasma & receiver:

Toggle left button until the left led shows “U2”

Toggle right button until the right led shows “1C”

Selecting Cable box to game room RPTV & receiver:

Toggle left button until the left led shows “U3”

Toggle right button until the right led shows “1C”

Selecting Xbox to kitchen LCD:

Toggle left button until the left led shows “U4”

Toggle right button until the right led shows “2O”

Step 5:  Using ZeroKey IR learning for remote control

The IR interface of the CRM88 is very similar to the front panel control.  We are just substituting a unique IR code (or remote button) for each of the left and right LED possible values.  For CRM84, this gives us 25 discreet codes in basic scheme:

Output select codes:  U1 - U8, CF           (9)

Input select codes: 1C – 8C, 1O – 8O     (16)

To control the CRM88, we can to map each of these discreet codes to a separate key on our remote.  However, since in our setup we are only using inputs 1C, 2O, 3O, 4O and 5 outputs we only need to use nine remote keys.

We are going to start by finding our receiver universal remote.  Like most universal remotes, it can control several devices.  In our system, we don’t have a VCR so we are going to use those codes to control CRM84.

Remote (VCR selected) key 1 => output select 1 (projector)

Remote (VCR selected) key 2 => output select 2 (plasma)

Remote (VCR selected) key 3 => output select 3 (RPTV)

Remote (VCR selected) key 4 => output select 4 (LCD)

Remote (VCR selected) key 5 => output select 5 (CRT)

Remote (VCR selected) key 6 => input select 1C (cable)

Remote (VCR selected) key 7 => input select 2O (xbox)

Remote (VCR selected) key 8 => input select 3O (dvd)

Remote (VCR selected) key 9 => input select 4O (pvr)

For each of these remote keys, we need to program the CRM88 to learn the transmitted code.  Keep in mind it is the CRM88 that is doing the recording, so don’t try to put the remote in “learn” mode.  Here are the steps for each key:

For the output selects (1,2,3,4,5)

Toggle the left front panel button until the correct output is displayed

Press and hold the left front panel button for 6 seconds

Release the left front panel button

The left LED should now be flashing, it is ready to record

Press the correct key on the remote (see above table)

The left LED should stop flashing, this key is stored

For the input selects (1C-4O)

Toggle the right front panel button until the correct input is displayed

Press and hold the right front panel button for 6 seconds

Release the right front panel button

The right LED should now be flashing, it is ready to record

Press the correct key on the remote (see above table)

The right LED should stop flashing, this key is stored

After storing all 9 remote keys, you can now control CRM88 with that remote.

Selecting DVD to projector:

Make sure universal remote is on “VCR” setting

Press remote button 1  (select projector)

Press remote button 8  (select DVD)

Selecting Cable box to plasma:

Make sure universal remote is on “VCR” setting

Press remote button 2  (select plasma)

Press remote button 6  (select Cable)

Selecting Xbox to LCD:

Make sure universal remote is on “VCR” setting

Press remote button 4  (select LCD)

Press remote button 7  (select Xbox)

If you ever need to change remotes, you can use the config clear command (“CFC” on LED and hold right button for 6 seconds) to reset all the codes at once.

This completes installation and setup of the CRM88.  If you have further questions please check our FAQ forum at video-storm.com for the most recent information.

Rear panel diagram (8x8 mode):